Ispit iz Računarskih Osnova i Primene Interneta
28/06/2006
1. (10p) Prijemnik prima streaming audio od servera i pušta ga brzinom od 100Kb/s.    
Maksimalni protok podataka je 128Kb/s, a kašnjenje   kroz mrežu je konstantno i isto u oba smera i iznosi 50ms. Ako je dimenzija bafera u prijemniku 10KB odrediti gornji i donji prag u baferu.

      2. (10p) Signal na telefonskoj liniji (4kHz propusni opseg) se odabira prema
             Nikvistovom kriterijumu i kvantizuje sa 16 nivoa u digitalni signal.
             Koliki je bitska brzina digitalnog signala?
3. (15p) Šest tokova prolazi kroz bafer na izlaznom linku 1000Base-LX Ethernet   sviča. Njihove ponudjene bitske brzine su 100Mb/s, 100Mb/s, 1Gb/s, 1Gb/s, 100Mb/s, 1Gb/s, a dodeljene težine su 2, 12, 2, 6, 8, 10 respektivno. Ako se podaci tokova šalju na link prema algoritmu za fer servis, izračunati sa kojom bitskom brzinom se svaki tok opslužuje. 
4. (15p) Odrediti stablo po kojima se prenose paketi u lokalnoj mreži koja se sastoji iz 
     kompletnih mostova B1, B2, … koje su povezani linkovim zadatih težina. Navesti    
     korake STA (“Spanning Tree” Algoritma) po kojem se formira stablo. Most BX, 
     ima identifikator (ID) jednak X. Broj pored nekog porta označava identifikator tog 
     porta, dok veći zadebljani broj pored linka označava težinu (cenu) portova
     koje taj link povezuje.

[image: image1]
      5.  (20p) Ana je prmetila da je njen privatni RSA ključ (d1,n1) isti kao Brankov javni 
          RSA ključ (e2,n2), odnosno da je d1=e2 i n1=n2. Da li Ana treba da promeni svoj 
          privatni i javni RSA ključ i zašto? 
    6. (20p) Lokalnoj mreži na slici treba dodeliti IP adrese. Koji je minimalni opseg    
         adresa potreban? Ako je 128.64.120.0 prva adresa na raspolaganju, dodeliti adrese 
         ruterima A, B, C i hostovima na slici?  


[image: image2]
7. (15p) Opisati primere protokola enkripcije i autentifikacije. Objasniti njihovu svrhu. 
8. (15p) Koje su prednosti i mane bežičnih u odnosu na žične komunikacione medije. 
[image: image3.emf]   

A F 





2





2




















2








�B











3








2





�








2





A








C





1





B3





�3





�2





�3





�1





�1





�1





�2





�2





�2





�6





�B4





�2





�B9





�B7





�B6





B8





�





�2





B5





B2





1





2





1





2





3





3








1





2





3





2











B1





1





1





1





1





1





3





2





3








[image: image4.emf] 

A 

[image: image5.emf] 

A 

[image: image6.emf] 

A 

[image: image7.emf] 

A 

[image: image8.emf] 

A 

[image: image9.emf] 

A 

[image: image10.emf] 

A 

[image: image11.emf] 

A 

[image: image12.emf] 

A 

[image: image13.emf] 

A 

[image: image14.emf] 

A 

[image: image15.emf] 

A 

[image: image16.emf] 

A 

[image: image17.emf] 

A 

[image: image18.emf] 

A 

[image: image19.emf] 

A 

[image: image20.emf] 

A 

[image: image21.emf] 

A 

[image: image22.emf]   

A F 

