Veljko Milutinović

MTP:
Understanding the Essence

emilutiv@etf.bg.ac.yu

UNDERSTANDING THE MTP

Basic classification:

coarse grain (task level; switching to a new thread on a context switch)

versus

fine grain (instruction level; switching to a new thread every cycle)

Principal components:

Multiple activity specifiers (program counters, stack pointers, etc.)

Multiple register contexts

Thread synchronization mechanism

(memory tags, HEP; 2-way joins, Monsoon; futures, MASA; etc.)

Fast context switch (Iannucci; Culler; etc.)

Differences between a thread and a process:

Thread may be directly supported at the architecture level

(start/suspension/continuation may be implemented in the ISA)

Process is implemented in the operating system layer

(start/suspension/continuation implemented in software)

COARSE-GRAINED MULTITHREADING

THE FIRST MULTITHREADING PROJECT: HEP

The first commercial MIMD based on multithreading (Denelcor, Inc., 1978)

Up to 16 PEMs (each one running up to 8 user and 8 supervisory threads)

and a number of DMMs (with dataflow heritage)

on a multistage ICN (any memory location accessible to any processor)

OTHER MULTITHREADING PROJECTS: 90’s

Tera (Smith, Tera)

Monsoon (Arvind, MIT and Motorola)

*T (Arvind, MIT and Motorola)

Super Actor Machine (Gao, McGill)

EM-4 (Sakai/Yamaguchi/Kodama, ETL)

MASA (Halstead/Fujita, Multilisp)

J-Machine (Dally, MIT)

Alewife (Agarwal, MIT)

Figure MTPU1: Structure of the HEP multiprocessor system (source: [Iannucci94])

Legend:

PEM—Processing Element Module

DMM—Data Memory Module

PSU—Packet Switch Unit

LAP—Local Access Path

ICN—Interconnection Network

Figure MTPU2: Structure of the HEP processing element module (source: [Iannucci94])

Legend:

FU—Functional Unit,

FP—Floating-Point.

FINE-GRAINED MULTITHREADING

TRADITIONAL FINE-GRAINED MULTITHREADING: TFM

In traditional fine-grain multithreading:

only one thread issuing instructions in each cycle.

SIMULTANEOUS MULTITHREADING: SMT

Several independent threads issuing instructions simultaneously

to multiple functional units of a superscalar (in a single cycle).

Higher potentials for utilization of resources in a wide-issue processor.

Throughput on an 8-issue processor:

4 times of a superscalar and 2 times of a fine-grained multithread,

because both horizontal and vertical waste are attacked simultaneously

Figure MTPU3: Empty issue slots: horizontal waste and vertical waste (source: [Tullsen95])

Legend: Self-explanatory

Superscaling not efficient for vertical waste;

multithreading not efficient for horizontal waste;

the SMT is never not efficient!

| Source of Wasted Issue Slots | Possible Latency-Hiding or Latency-Reducing Techniques |
| --- | --- |
| instruction TLB miss, data TLB miss | decrease the TLB miss rates (e.g., increase the TLB sizes); hardware instruction prefetching; hardware or software data prefetching; faster servicing of TLB misses |
| I cache miss | larger, more associative, or faster instruction cache hierarchy; hardware instruction prefetching |
| D cache miss | larger, more associative, or faster data cache hierarchy; hardware or software prefetching; improved instruction scheduling; more sophisticated dynamic execution |
| branch misprediction | improved branch prediction scheme; lower branch misprediction penalty |
| control hazard | speculative execution; more aggressive if-conversion |
| load delays (first-level cache hits) | shorter load latency; improved instruction scheduling; dynamic scheduling |
| short integer delay | improved instruction scheduling |
| long integer, short fp, long fp delays | (multiply is the only long integer operation, divide is the only long floating point operation) shorter latencies; improved instruction scheduling |
| memory conflict | (accesses to the same memory location in a single cycle) improved instruction scheduling |

Figure MTPU4: Causes of wasted issue slots and related prevention techniques (source: [Tullsen95])

Legend:

TLB—Translation Lookaside Buffer.

All these techniques have to be utilized properly,

before the effects of SMT can be studied.

| Purpose of Test | Common Elements | Specific Configuration | T |
| --- | --- | --- | --- |
| Unlimited FUs: | Test A: FUs = 32 | SM: 8 thread, 8-issue | 6.64 |
| equal total issue bandwidth, | IssueBw = 8, RegSets = 8 | MP: 8 1-issue | 5.13 |
| equal number of register sets | Test B: FUs = 16 | SM: 4 thread, 4-issue | 3.40 |
| (processors or threads) | IssueBw = 4, RegSets = 4 | MP: 4 1-issue | 2.77 |
|  | Test C: FUs = 16 | SM: 4 thread, 8-issue | 4.15 |
|  | IssueBw = 8, RegSets = 4 | MP: 4 2-issue | 3.44 |
| Unlimited FUs: | Test D: | SM: 8 thread, 8-issue, 10 FU | 6.36 |
| Test A, but limit SM to 10 FUs | IssueBw = 8, RegSets = 8 | MP: 8 1-issue procs, 32 FU | 5.13 |
| Unequal issue BW: | Test E: FUs = 32 | SM: 8 thread, 8-issue | 6.64 |
| MP has up to four times | RegSets = 8 | MP: 8 4-issue | 6.35 |
| the total issue bandwidth | Test F: FUs = 16 | SM: 4 thread, 8-issue | 4.15 |
|  | RegSets = 4 | MP: 4 4-issue | 3.72 |
| FU utilization: | Test G: FUs = 8 | SM: 8 thread, 8-issue | 5.30 |
| equal FUs, equal issue bw, unequal reg sets | IssueBw = 8 | MP: 2 4-issue | 1.94 |

Figure MTPU5: Comparison of various (multithreading) multiprocessors and an SMT processor (source: [Tullsen95])

Legend:

T—Throughput (instructions/cycle)

Current microprocessors are mostly 4-issue superscalars;

potentially, SMT leads to 8-issue and 16-issue next-gen superscalars.

REFERENCES

[Iannucci94]	Iannucci, R. A., Gao, G. R., Halstead, R. H. Jr., Smith, B.,
Multithreaded Computer Architecture:
A Summary of the State of the Art,
Kluwer Academic Publishers, Boston, Massachusetts, USA, 1994.

[Tullsen95]	Tullsen, D. M., Eggers, S. J., Levy, H. M.,
“Simultaneous Multithreading: Maximizing On-Chip Parallelism,”
Proceedings of the ISCA-95, Santa Margherita Ligure, Italy, 1995, pp. 392–403.

Veljko Milutinović

MTP:
State of the Art

emilutiv@etf.bg.ac.yu

AN INDUSTRIAL MTP PROCESSOR

Problem:

Memory accesses are starting to dominate execution time of uniprocessors

Solution:

Coarse grained uniprocessor multithreading in the IBM environment

Conditions:

Object oriented programming for on-line transactions processing

Reference:

[Eickmeyer96]	Eickmeyer, R. J., Johnson, R. E., Kunkel, S. R., Liu, S.,
Sqillante, M. S.,
“Evaluation of Multithreaded Uniprocessor
for Commercial Application Environments,”
Proceedings of the ISCA-96, Philadelphia, Pennsylvania,
May 1996, pp. 203–212.

AN ACADEMIC SMT PROCESSOR

Problem:

Extending a conventional wide-issue superscalar to SMT

Solution:

Combining of the following principles:

(a) minimizing the changes to the conventional superscalar architecture

(b) making the single thread case to be suboptimal (2%)

(c) achieving maximal throughput when running multiple threads

Throughput improvement: 5.4 (smt) over 2.5 (equivalent superscalar)

Conditions:

Eight threads with a modified Multiflow compiler

Reference:

[Tullsen96]	Tullsen, D. M., Eggers, S.J., Emer, J. S., Levi, H. M., Lo, J.L.,
Stamm, R. L.,
“Exploiting Choice: Instruction Fetch and Issue
on an Implementable Simultaneous Multithreading Processor,”
Proceedings of the ISCA-96, Philadelphia, Pennsylvania,
May 1996, pp. 191–202.

Veljko Milutinović

MTP:
IFACT

emilutiv@etf.bg.ac.yu

Combining Catalytic Migration
and Catalytic Reincarnation

Essence:

- Research in progress at the University of Belgrade;
supported by FNRS

- Combining the best of the two most promising approaches:
Catalytic Migration and Catalytic Reincarnation

- Two research activities working in parallel
numerical domain (Davidovic) and symbolic domain (Janicijevic)

- Speedup over traditional SMT
is application dependent

References:

[Milutinovic96a]	Milutinovic, V.,
“Some Solutions for Critical Problems
in Distributed Shared Memory,”
IEEE TCCA Newsletter, September 1996.

[Milutinovic96b]	Milutinovic, V.,
“The Best Method for Presentation of Research Results,”
IEEE TCCA Newsletter, September 1996.